🚗 EV Fault Detection Using SOC + LSTM

This project detects battery faults in Electric Vehicles (EVs) by analyzing State of Charge (SOC) behavior.

It uses:

- Real SOC data from fresh (healthy) and aged (faulty) Li-ion cells

- SOC estimation through Coulomb Counting

- Preprocessing (smoothing / ANC-like filtering)

- Deep learning using LSTM (Long Short-Term Memory) sequence modeling

- Classification into Normal (Fresh) and Fault (Aged) battery states

1. Project Objective

To build a diagnostic model that can automatically detect whether an EV battery is healthy or faulty based on its SOC pattern over time, using a deep-learning model (LSTM).

EV batteries degrade over time due to:

- Increased internal resistance

- Loss of lithium inventory

- Poor voltage-current response

These degradations change the SOC curve, which is what the LSTM learns to differentiate.

2. Input Data

You have provided two CSV files:

- Fresh Battery Data

➡ Contains voltage, current, and time for a new cell

- Aged Battery Data

➡ Represents degraded or faulty battery behavior

Your code loads these two datasets:

aged  = readtable(aged_path);

fresh = readtable(fresh_path);

You extract:

- Time

- Current

- Voltage

These will be used to calculate SOC.

3. SOC Estimation — Coulomb Counting

SOC is estimated using:

SOC(t)=SOC(0)-1Cnom∫I(t) dt

In MATLAB:

SOC_fresh = 1 - cumsum(I_fresh .* dt_fresh) / C_nom;

SOC_aged  = 1 - cumsum(I_aged  .* dt_aged ) / C_nom;

✔ Coulomb counting advantages:

- Simple

- Works with minimal sensors

✔ Why SOC reveals battery health:

Aged batteries show:

- faster SOC drop

- noisier SOC curves

- different response under same load

Thus, SOC is a behavioral fingerprint of battery health.

4. Combining + Labeling Data

Health classes:

- Fresh → Label 1

- Aged → Label 2

soc_raw = [SOC_fresh(:); SOC_aged(:)];

labels_raw = [ones(length(SOC_fresh),1); 2*ones(length(SOC_aged),1)];

Then you smooth the SOC using movmean (like adaptive noise cancellation):

soc_anc = movmean(soc_raw, 5)';

This removes spikes but keeps trend patterns.

5. Creating LSTM Sequences

LSTMs require sequences of equal length.

You use:

- sequence length = 50 time samples

- step size = 10

Sliding window segmentation:

SOC: |----50----|----50----|----50----| ...

step:      10         10         10

Each segment becomes one training example.

X{idx} = reshape(seq(:)', [1, sequence_length]);

Labels assigned using mean of that segment.

6. Preparing Train/Test Datasets

You split 70% for training and 30% for testing:

train_idx = idx(1:round(0.7*N));

test_idx  = idx(round(0.7*N)+1:end);

7. LSTM Network Architecture

✔ Your LSTM Model:

sequenceInputLayer(1)

lstmLayer(50,'OutputMode','last')

fullyConnectedLayer(numClasses)

softmaxLayer

classificationLayer

Why LSTM?

LSTMs are excellent for:

- time series

- trends

- long-term behavior

- pattern recognition

Your SOC is time-varying → perfect for LSTM.

8. Training the Model

You train using stochastic gradient descent (sgdm):

[net, info] = trainNetwork(XTrain, YTrain, layers, options);

During training the model learns:

- fresh batteries have smooth, steady SOC drops

- aged batteries have unstable, fast decays or voltage-related distortions

9. Testing & Accuracy

Model predicts class for each test sequence:

YPred = classify(net, XTest);

acc = mean(YPred == YTest);

Prints:

Test Accuracy
 MAE, RMSE, MAPE (error metrics)

High accuracy means the LSTM has successfully learned the difference in SOC behavior.

10. Plotting SOC Signals

Two plots:

- Raw SOC (fresh + aged combined)

- Smoothed SOC

This helps visualize differences in dynamic behavior.

11. What the Model is Detecting Internally

The LSTM learns hidden features such as:

Fresh battery:

- Gradual SOC drop

- Low noise

- Predictable voltage response

Aged battery:

- Faster SOC decline

- Voltage sag causes sudden SOC dips

- Irregular patterns

These patterns help classify between Normal vs Faulty.

12. Practical Applications

This model can be used in:

✔ EV Battery Health Monitoring

Detects early faults to prevent breakdown.

✔ Predictive Maintenance

Identify battery aging without opening the pack.

✔ BMS (Battery Management System) Diagnostics

A real-time LSTM in embedded system for health state prediction.

13. Advantages of Your Approach

- Uses minimal sensors (only current & time)

- Fast SOC estimation

- Data-driven, no complex battery modeling

- LSTM captures nonlinear degradation behavior

14. Possible Improvements

To enhance accuracy, you can add:

- Temperature as an input feature

- Voltage-based SOC correction

- GRU / BiLSTM variants

- Feature fusion (I, V, T, SOC)

- Data augmentation

- Real-time streaming architecture

✅ Final Summary

This project builds a complete EV battery fault detection system using SOC + LSTM.
It processes raw current/time data, computes SOC, segments into sequences, and trains a neural network to detect whether the battery is fresh or aged.

This approach is powerful because SOC behavior directly reflects internal degradation, and LSTM is well-suited for capturing time-dependent patterns.

That image shows a comparison of two State-of-Charge (SOC) signals, likely from a battery or energy storage system, over a 2000-second time period.

Here is a simple explanation of what the graphs illustrate:

🔋 Top Graph: Raw SOC Signal (Fresh+Fault Combined)

- This shows the original, unfiltered State-of-Charge data.

- The SOC is generally very high (close to 1, or 100%), which is typical for a fully charged or nearly fully charged battery.

- Around the 1000-second mark, there is a sudden, sharp dip or spike. This is likely the "Fault" part of the signal, which could be:

  - An actual, sudden change in the battery's state.

  - A measurement noise/glitch or a data error that needs to be corrected.

📉 Bottom Graph: Preprocessed SOC (ANC-smoothed)

- This shows the SOC signal after a smoothing technique called "ANC-smoothed" (likely referring to an Adaptive Noise Cancellation or similar filtering method) has been applied.

- The overall signal is still close to 1, but the y-axis scale is slightly different, showing the signal is more focused in the range of 0.997 to 1.

- Crucially, the sharp dip at 1000 seconds is still present, but it appears slightly less abrupt and potentially 'cleaner' or more defined. The ANC-smoothing has not eliminated the fault but has processed the signal around it, likely removing high-frequency noise that was present in the raw signal.

📝 Key Takeaway

The image demonstrates the effect of signal processing (ANC-smoothing) on battery data. The goal of this process is to remove unwanted noise from the "Raw" signal to get a clearer, more reliable "Preprocessed" signal, especially when looking for real events like the fault occurring at  seconds.

This image shows the training progress of a Machine Learning model (likely a neural network). The graph tracks two key measurements, Accuracy and Loss, over time as the model learns.

Here is a simple breakdown:

📈 What the Graphs Mean

1. Accuracy (Top Graph)

- What it is: The percentage of correct predictions the model makes. Higher is better.

  - Training Accuracy (Blue Line): How well the model performs on the data it is actively learning from.

  - Validation Accuracy (Orange Line): How well the model performs on new, unseen data (the "validation set"). This is the most important measure of how good the model is in the real world.

- What you see: Both lines are increasing over time, which is good! The model is learning.

- The Problem: The lines are very wiggly or "oscillating." This means the model's performance jumps up and down a lot with each update.

2. Loss (Bottom Graph)

- What it is: A number that measures the model's error (how wrong its predictions are). Lower is better.

  - Training Loss (Blue Line): The error on the data the model is learning from.

  - Validation Loss (Orange Line): The error on the new, unseen data.

- What you see: Both lines are decreasing over time, which is good! The model is reducing its error.

- The Problem: Just like accuracy, the lines are oscillating dramatically.

🛑 Why the Lines are Wiggly (Oscillation)

The intense up-and-down movement is usually caused by the model taking steps that are too big during the learning process.

Imagine the model is a person trying to walk down a hill (to find the lowest Loss).

- Ideal Step: The person takes small, gentle steps toward the very bottom. Loss goes down smoothly.

- Too Big a Step (Oscillation):

  - The person takes a giant leap that completely overshoots the bottom of the hill.

  - Now they're on the other side, and the Loss goes up dramatically.

  - In the next step, they try to correct by leaping back, but they overshoot again, so the Loss goes up again.

  - This back-and-forth leaping (overshooting) is what causes the oscillating pattern you see.

Common Cause: High Learning Rate

In machine learning terms, the size of this "step" is called the Learning Rate. The most common reason for this kind of intense oscillation is a Learning Rate that is too high. The model is making updates to its internal parameters that are too aggressive.

🛠️ How to Fix It

The main fix for a highly oscillating training curve is to:

- Reduce the Learning Rate: Make the model take smaller, more careful steps toward the optimal solution.

- Use a Learning Rate Schedule: Program the learning rate to start a little higher but automatically get smaller (decay) as training progresses.

The goal is to get a smoother curve that steadily improves (Accuracy goes up, Loss goes down) without the big jumps.

Machine Learning Model Training

The images show a model (a computer program) being trained to do a task, likely a prediction or classification, possibly related to the battery data shown in the first image.

1. Training Progress (Graph)

- What it shows: How well the model is learning over 20 training cycles (Epochs).

- The Problem: Both the Accuracy (top, blue line) and Loss (bottom, orange line) are violently wiggly (oscillating).

  - Accuracy measures how often the model is correct (should go up).

  - Loss measures the model's error (should go down).

- Why it's Wiggly: The model is taking steps that are too big during its learning process, which is indicated by the Learning rate being set to a constant $\mathbf{0.01}$ (see "Other Information" box). It keeps overshooting the correct answer, causing its performance to jump up and down.

2. Command Window (Training Data)

- What it shows: The raw numbers as the model trained.

- Key Numbers:

  - The Mini-batch Loss is consistently high, hovering around $\mathbf{0.68}$ to $\mathbf{0.70}$.

  - The Mini-batch Accuracy is low, bouncing between $43.75\%$ and $56.25\%$.

  - The Base Learning Rate is $\mathbf{0.01}$, which confirms the cause of the wobbly graphs.

- Final Result: The Test Accuracy is only $\mathbf{42.3729\%}$. This is a very poor result, meaning the model is only slightly better than a random guess at its task.

3. Raw Signal (Potential Task Data)

- What it shows: A State-of-Charge (SOC) signal, likely from a battery, over time.

- Key Feature: There is a huge, sudden fault or glitch (a big dip) in the signal around $\mathbf{1000}$ seconds.

- Connection to ML: The low-performing machine learning model ($\approx 42\%$ accuracy) was likely trying to learn to detect or predict this kind of fault, but it failed to learn correctly because of the unstable training process.